COMPARATIVE STUDY OF PIPELINED MULTIPLIER ARCHITECTURES ON FPGA AND ITS APPLICATION IN DSP ACCELERATORS

Internship Technical Report

1. Abstract

High-speed multiplication is one of the fundamental computational requirements of digital signal processing systems. Multipliers are frequently located on critical datapaths and therefore have a direct influence on the achievable operating frequency, throughput, area, and power consumption of a digital system.

This internship project investigated the design, implementation, verification, pipelining, and FPGA benchmarking of multiple multiplier and adder architectures. The work was organized into three major phases.

Phase 1 investigated plain non-pipelined multiplier architectures and developed a common RTL and verification framework. Architectures including Braun, Row Adder, Carry Save, Wallace Tree, Dadda, Booth, Baugh-Wooley, Vedic, and a Greedy Column Reduction architecture were studied and implemented.

Phase 2 investigated pipelined multiplier architectures. Different register-placement strategies were evaluated, including input/output registers, every-stage pipelining, and every-two-stage pipelining with respect to the Wallace Multiplier. Particular attention was given to the relationship between pipeline depth, register count, critical-path delay, throughput, and FPGA resource utilization.

During this phase, an important verification problem was encountered in which pipelined implementations produced incorrect outputs despite the corresponding non-pipelined architectures producing correct results. The investigation revealed that some signals entering a later stage had crossed different numbers of pipeline registers. This created temporal misalignment between signals that were mathematically supposed to be combined. The problem was corrected by ensuring that all signals required at a particular stage were synchronized to the same logical pipeline depth.

Phase 3 expanded the investigation to carry-propagation architectures. Multiple adders were implemented and tested, including Carry Lookahead, Carry Skip, Carry Select, Kogge-Stone, Brent-Kung, Han-Carlson, Sklansky, Ladner-Fischer, Knowles, Sparse Kogge-Stone, Sparse Brent-Kung, Ling, Manchester Carry, and Radix-4 Kogge-Stone variants.

All primary FPGA benchmarking was performed using Quartus II 13.0sp1 targeting the Cyclone II DE2 FPGA (EP2C35F672C7). The same experimental environment was maintained wherever possible so that architectural comparisons were meaningful.

The project demonstrated that multiplier performance cannot be understood solely from theoretical logic depth. FPGA-specific resources, routing, dedicated carry structures, synthesis optimization, fan-out, register placement, and architecture-to-device mapping all strongly influence the measured result.

The project ultimately established a reusable research framework for structural multiplier and adder exploration and provided the foundation for subsequent work toward a dedicated DSP accelerator architecture.

2. Introduction

Digital signal processing systems perform large numbers of arithmetic operations, frequently involving multiplication and addition. Operations such as multiply-accumulate, filtering, convolution, correlation, and Fourier transforms depend heavily on the efficiency of arithmetic datapaths.

A straightforward binary multiplier can generate a large number of partial products and then reduce them using addition. As the operand width increases, the number of partial products and the amount of carry propagation increase rapidly.

Consequently, multiplier architecture becomes an important design variable.

Different architectures attempt to solve this problem in different ways:

- sequentially adding partial products,

- reducing partial products using compressor structures,

- organizing reduction operations into trees,

- encoding the multiplier operand to reduce the number of partial products,

- using signed multiplication techniques,

- or directly constructing product bits through alternative reduction strategies.

This project investigates these approaches experimentally rather than relying exclusively on theoretical complexity.

3. Project Objectives

The major objectives of the internship were:

- Study multiple digital multiplier architectures.

- Develop parameterized structural RTL implementations.

- Investigate pipelined multiplier architectures.

- Study the effect of pipeline-register placement on performance and area.

- Identify critical datapath bottlenecks.

- Study final carry-propagate adder architectures.

- Benchmark architectures under identical FPGA conditions.

- Investigate architectural trade-offs between speed, area, power, and complexity.

- Develop a foundation for future DSP accelerator development.

4. Overall Project Timeline

The internship research was organized into three major implementation phases.

| Phase | Dates | Primary Work |
| --- | --- | --- |
| Phase 1 | 24 June – 2 July | Multiplier architectures |
| Phase 2 | 3 July – 18 July | Pipelined multipliers |
| Phase 3 | 18 July – 26 July | Adder architectures |

The three phases were deliberately sequential.

Phase 1 established the multiplier architectures.

Phase 2 investigated how those architectures behave when pipelined.

Phase 3 then attacked one of the major bottlenecks identified during Phase 2: the final carry-propagate addition.

5. Phase 1 — Multiplier Architecture Investigation

5.1 Objective

The first phase established a common framework for comparing different multiplier architectures.

Instead of implementing a single multiplier and optimizing it in isolation, multiple architectures were constructed using a common methodology.

This made it possible to investigate how structural differences affect:

- area,

- delay,

- frequency,

- power,

- logic utilization,

- and scalability.

5.2 Architectures Investigated

The multiplier architectures included:

- Braun multiplier

- Row Adder multiplier

- Carry Save multiplier

- Wallace Tree multiplier

- Dadda multiplier

- Booth multiplier

- Baugh-Wooley multiplier

- Vedic multiplier

- Greedy Column Reduction multiplier

The architectures differ significantly in how partial products are generated and reduced.

6. Partial Product Generation

The first major operation in multiplication is partial-product generation.

For an N×N multiplier, N² elementary partial-product terms can exist in the unsigned case.

These terms are organized according to their binary weight.

For example:

Partial Products

PP

PP

PP

PP

PP

PP

Each diagonal corresponds to a particular product-bit position.

The subsequent reduction architecture determines how efficiently these partial products can be transformed into the final product.

7. Partial Product Reduction

The principal difference between several multiplier architectures lies in their reduction strategy.

A Row Adder architecture performs relatively sequential accumulation.

Wallace and Dadda architectures instead reduce partial-product columns through compressor structures.

The general concept is:

Many Partial Products

↓

Reduction

↓

Fewer Rows

↓

Two Rows

↓

Final Carry-Propagate Addition

The final step became particularly important during the later phases of the project.

8. Greedy Column Reduction Architecture

One of the major architectural explorations was a Custom Greedy Column Reduction architecture.

Instead of using a conventional horizontal reduction-tree structure, the architecture performs reduction locally and progressively across product columns.

For a column containing three or more active bits, a full adder can reduce three bits to a sum bit and a carry bit.

For a column containing two active bits, a half adder can be used.

The carry is then introduced into the next column.

A significant architectural property of this approach is that the reduction process directly resolves the individual product-bit columns.

Consequently, unlike conventional Wallace/Dadda-style architectures that reduce the matrix to two rows and then require a separate final carry-propagate adder, the Custom Greedy architecture can directly construct the product vector.

The final RCA stage was therefore unnecessary in this architecture.

This created an important speed-versus-area trade-off:

The architecture can use more reduction hardware while potentially avoiding the delay of a conventional final carry-propagate adder.

The measured FPGA behavior showed that this architectural difference was significant enough to justify further investigation.

9. Phase 2 — Pipeline Investigation

9.1 Motivation

The combinational delay of a multiplier limits its maximum operating frequency.

A simplified relationship is:

Fmax=1Critical Path Delay

Pipelining breaks a long combinational path into multiple shorter paths.

Instead of requiring one clock period to accommodate the entire computation, the computation is divided across multiple cycles.

10. Pipeline Methodology

Several pipeline configurations were investigated.

The principal strategies included:

- input register,

- PPG register,

- every-stage pipeline,

- every-two-stage pipeline,

- output register.

The common experimental configuration used for later comparisons was:

Input Register

↓

Multiplier

↓

Register every 2 stages

↓

Final computation

↓

Output Register

This provided a consistent basis for comparing architectures.

11. Pipeline Stage Versus Register Count

An important lesson from the project was that the number of physical registers does not automatically represent the logical pipeline latency.

A register bank containing hundreds of flip-flops can represent one pipeline boundary.

Conversely, a signal may have to cross several different register boundaries before reaching its consuming logic.

Therefore:

Pipeline latency is determined by sequential boundaries crossed by the data path, not simply by the number of individual flip-flops.

This distinction became particularly important during verification.

12. Pipelined Multiplier Verification

The original non-pipelined testbench could compare the multiplier output immediately after applying the inputs.

This assumption becomes invalid when pipeline registers are introduced.

For a pipelined design:

Input(t)

↓

Stage 1

↓

Register

↓

Stage 2

↓

Register

↓

Output(t+2)

Therefore the expected result must also be delayed appropriately.

A golden reference pipeline was developed.

The testbench effectively performed:

A × B

↓

Golden Model

↓

Expected Pipeline

↓

Compare with DUT

Exhaustive testing was used wherever practical.

13. The Pipeline Synchronization Failure

One of the most important debugging experiences occurred after the testbench synchronization had already been corrected.

The testbench correctly accounted for pipeline delay, yet the pipelined multiplier still produced incorrect numerical results.

The non-pipelined version produced correct results.

This strongly suggested that the arithmetic architecture itself was not necessarily defective.

The investigation eventually identified the real problem:

Different signals entering the same later-stage computation had crossed different numbers of pipeline registers.

For example:

PPG

|

|────────────────── Signal A

|

| Adder   Register   signal B

↓

Later Stage

Signal A could therefore arrive earlier than signal B.

The later adder was mathematically correct but was receiving operands belonging to different input transactions.

The result was consequently incorrect.

The solution was to ensure that every signal required by a particular stage was synchronized to the same logical pipeline depth, including signals that had not been consumed by earlier reduction stages.

This revealed an important general principle:

A pipeline boundary applies to the complete set of data required by the future computation, not merely to the signals that happened to pass through an adder.

14. Pipeline Register Overhead

The synchronization requirement introduced another important observation.

In a conventional horizontal reduction tree, many signals naturally belong to the same stage.

In the Custom Greedy architecture, however, the computation is strongly column-oriented.

Some partial-product bits may be consumed early while others survive for several stages.

This can cause a pipeline register to preserve a relatively large set of signals merely so that all paths remain temporally aligned.

Consequently:

Useful arithmetic hardware

↓

Pipeline registers

↓

Synchronization overhead

can become a significant area component.

This led to a separate research direction concerning delay-register optimization.

15. Every-Stage Versus Every-Two-Stage Pipelining

Two important strategies were compared:

Every-stage

Stage

↓

REG

↓

Stage

↓

REG

↓

Stage

↓

REG

Every-two-stage

Stage

↓

Stage

↓

REG

↓

Stage

↓

Stage

↓

REG

The expectation was that every-stage pipelining would necessarily provide a substantially higher frequency.

However, the experimental results showed that the improvement eventually became small.

This demonstrated a phenomenon that can be described as pipeline saturation.

Once the major combinational bottleneck has been sufficiently shortened, adding additional registers does not necessarily improve Fmax.

Other factors become dominant:

- register overhead,

- routing,

- clock-to-Q delay,

- setup time,

- FPGA interconnect,

- dedicated carry structures,

- synthesis optimization,

- and remaining unpipelined logic.

16. Final Adder Bottleneck

Conventional Wallace and Dadda multipliers eventually reduce their partial products to two rows.

The remaining operation is:

Two Rows

↓

Carry Propagate Adder

↓

Product

Therefore, pipelining the reduction tree does not automatically eliminate the delay of the final adder.

With an RCA:

A0 + B0

↓ carry

A1 + B1

↓ carry

A2 + B2

↓ carry

...

the carry must propagate through the datapath.

This eventually made the final adder a significant critical-path candidate.

This observation motivated Phase 3.

17. Phase 3 — Adder Architecture Investigation

Phase 3 began on 18 July and was completed on 26 July.

The objective was to investigate alternative carry-propagation architectures that could replace the RCA in the multiplier datapath.

The primary experimental environment remained:

- Quartus II 13.0sp1

- Cyclone II DE2

- EP2C35F672C7

- 16×16 pipelined Wallace multiplier

- Input register

- Every-two-stage pipeline

- Output register

This common environment allowed the different adder architectures to be compared under the same system-level conditions.

18. Adder Architectures Implemented

The implemented architectures included:

- Carry Lookahead Adder

- Carry Skip Adder

- Carry Select Adder

- Kogge-Stone Adder

- Brent-Kung Adder

- Han-Carlson Adder

- Sklansky Adder

- Ladner-Fischer Adder

- Knowles Adder

- Sparse Kogge-Stone

- Sparse Brent-Kung

- Ling Adder

- Manchester Carry Adder

- Radix-4 Kogge-Stone

Multiple CLA implementations were also investigated, including block, hierarchical, and parallel-prefix forms.

19. Kogge-Stone

Kogge-Stone provides logarithmic carry propagation depth.

Its major advantage is low logical depth.

Its disadvantage is the large number of interconnections and associated routing requirements.

The implementation was modified and corrected so that it worked for arbitrary tested widths rather than only a restricted configuration.

The architecture therefore represents an important example of the distinction between:

theoretical logic depth

and

actual FPGA implementation cost.

20. Brent-Kung

Brent-Kung reduces the amount of wiring compared with Kogge-Stone.

Its forward tree calculates grouped prefixes, followed by a backward distribution tree.

This reduces wiring density but increases logical depth relative to Kogge-Stone.

The implementation was modified and corrected during development.

21. Han-Carlson

Han-Carlson was investigated as a hybrid between Kogge-Stone and Brent-Kung.

The architecture attempts to balance:

- speed,

- wiring,

- fan-out,

- and area.

It therefore represents a useful intermediate point in the architecture design space.

22. Carry Select — An Unexpected FPGA Result

Carry Select theoretically duplicates arithmetic hardware so that both possible carry-in cases can be computed in parallel.

Therefore, one might expect a large area penalty.

However, the FPGA results do not necessarily expose that theoretical hardware duplication directly.

The reason is that the FPGA synthesis tool can map simple logic efficiently into LUT resources.

Therefore:

A theoretical ASIC area penalty does not necessarily appear as the same area penalty on an FPGA.

This is an important limitation of FPGA benchmarking.

The measured FPGA result should therefore not be interpreted as proof that Carry Select is intrinsically smaller than other architectures.

The result instead demonstrates how strongly the target technology influences architecture-to-area mapping.

23. Sklansky and Ladner-Fischer — Nearly Identical Results

Theoretical descriptions suggest that Sklansky and Ladner-Fischer have different fan-out and wiring characteristics.

However, the Quartus implementation produced effectively identical reported results for the tested configuration.

This does not mean that the architectures are theoretically identical.

Rather, it demonstrates that synthesis can transform structurally different RTL descriptions into equivalent or near-equivalent FPGA implementations.

Therefore:

RTL topology does not necessarily equal final FPGA topology.

The FPGA synthesis tool is free to optimize, merge, restructure, and map logic according to the target device.

This is one of the strongest examples from the project of why architectural claims must be distinguished from implementation-specific observations.

24. Sparse Kogge-Stone — Area/Speed Trade-off

The Sparse Kogge-Stone implementation computes carries only at selected boundaries and allows local RCA blocks to handle the remaining regions.

The initial implementation constructed more of the Kogge-Stone structure than necessary and relied on synthesis to remove unused logic. This issue was corrected.

An additional assumption that the operand width must be divisible by the sparsity factor was also removed.

The corrected implementation supports arbitrary tested widths and sparsity factors.

The resulting architecture significantly reduced logic usage, while accepting some performance degradation due to the local ripple sections.

This is a useful example of an important engineering principle:

If optimization depends on synthesis to remove unnecessary hardware, the RTL itself may not yet represent the intended architecture.

25. Sparse Brent-Kung — An Especially Interesting Result

The Sparse Brent-Kung implementation produced one of the most interesting observations.

The initial implementation generated a ripple-like correction path.

For a 32-bit example with a sparsity factor of four, certain required intermediate carries were not generated directly by the sparse tree.

The missing carry relationships created a chain similar to:

27 → 23 → 19 → 15

which introduced ripple-like delay.

The architecture was subsequently modified so that the Brent-Kung backward tree continued far enough to generate the necessary sparse boundary terms before handing the remaining work to local RCA blocks.

The comparison produced:

| Configuration | LE | Combinational Functions | Fmax |
| --- | --- | --- | --- |
| Ripple Correction | 801 | 799 | 172.74 MHz |
| Backward Tree Correction | 793 | 792 | 159.87 MHz |

The backward-tree implementation therefore used fewer resources but was slower in the tested FPGA configuration.

At first glance this appears counterintuitive.

The likely explanation is FPGA-specific implementation.

The short ripple correction path may have been mapped efficiently onto the FPGA's dedicated carry resources, whereas the tree-based correction required general routing resources.

Therefore, a theoretically superior tree structure can lose to a ripple structure when the ripple path is short and the target FPGA provides dedicated carry hardware.

The result should not therefore be generalized to ASIC technology.

It is a particularly good example of:

Architecture × Technology = Actual Performance

rather than:

Architecture = Performance

26. Manchester Carry Adder

The Manchester Carry architecture was particularly interesting because it represents a transistor-level style of carry implementation.

The tested implementation showed very low LE utilization on the Cyclone II, but poor speed.

The reason is that the architecture's conceptual advantage is not naturally represented by FPGA LUT structures.

The implementation effectively maps poorly to the FPGA's architectural primitives.

Manchester Carry is therefore more interesting as an ASIC/transistor-level architecture than as an FPGA architecture.

The project documentation specifically identifies this distinction and notes that an ASIC-specific transistor-level implementation would be more appropriate for evaluating the architecture's real potential.

27. Ling Adder

The Ling modification reformulates carry equations using pseudo-carry concepts.

The objective is to reduce the effective logic required for carry computation.

However, the benefit is strongly technology-dependent.

The implementation observations indicate that Ling-style modifications are potentially more useful for ASIC implementations than for the FPGA environment used in this project.

This again demonstrates that an architecture cannot be evaluated independently of its implementation technology.

28. Radix-4 Kogge-Stone

A Radix-4 Kogge-Stone structure was also investigated.

Instead of combining information using binary prefix operations, a higher-radix prefix operation can reduce logical depth.

For a 16-bit datapath, the theoretical prefix depth becomes:

log₂(16) = 4

for radix-2,

whereas:

log₄(16) = 2

for radix-4.

The implementation achieved a reported 184.81 MHz result in the tested configuration.

The approach is particularly interesting for future ASIC exploration because higher-radix prefix cells can potentially be implemented using transistor-level techniques such as transmission-gate or pass-transistor structures.

29. FPGA Versus ASIC

One of the strongest conclusions of the internship is that FPGA measurements should not automatically be interpreted as ASIC conclusions.

An FPGA contains:

- LUTs,

- dedicated carry chains,

- fixed routing structures,

- dedicated registers,

- architecture-specific interconnect,

- synthesis and mapping optimizations.

An ASIC instead uses:

- standard cells,

- transistor-level structures,

- physical wires,

- parasitic capacitance,

- resistance,

- placement,

- routing,

- clock-tree effects,

- process-voltage-temperature variation.

Consequently, an architecture that performs poorly on an FPGA may become attractive in an ASIC.

Conversely, an architecture that performs exceptionally well on an FPGA may be benefiting from dedicated FPGA resources that do not exist in an ASIC.

This was repeatedly observed during the adder investigation.

30. Verification Infrastructure

Reusable testbench infrastructure was developed for the adders.

Two primary testing environments were implemented:

- exhaustive adder testing,

- randomized adder testing.

The architecture under test could be changed without rebuilding the complete verification environment.

The testbenches were placed in the adder verification framework and were successfully used across multiple configurations.

31. Experimental Philosophy

The project followed an important methodological principle:

Theory predicts the design space; synthesis and measurement determine the actual implementation result.

Theoretical architecture characteristics were therefore treated as hypotheses rather than guaranteed outcomes.

For example:

Theory

↓

Expected area/speed trade-off

↓

RTL implementation

↓

Synthesis

↓

Technology mapping

↓

Measured result

↓

Explanation

Unexpected results were investigated instead of being discarded.

This was one of the most important lessons of the project.

32. Major Lessons Learned

32.1 Logic depth is not the whole story

Two architectures with similar logical depth can have different measured frequencies because of routing, fan-out, and technology mapping.

32.2 More registers do not always mean higher Fmax

Once the dominant combinational delay is sufficiently divided, register and routing overhead can dominate.

32.3 Pipeline correctness requires temporal correctness

A mathematically correct datapath can produce incorrect results if signals belonging to different pipeline generations are combined.

32.4 FPGA carry chains matter

A short RCA section can sometimes outperform a theoretically more sophisticated tree because the FPGA provides dedicated carry resources.

32.5 FPGA synthesis can hide RTL differences

Structurally different RTL can become nearly identical after synthesis.

32.6 ASIC conclusions require ASIC experiments

Transistor-level architectures such as Manchester Carry and higher-radix structures cannot be fairly judged using FPGA LUT utilization alone.

33. Major Research Contributions

The completed internship work produced the following major contributions:

- Structural RTL implementations of multiple multiplier architectures.

- Reusable multiplier verification infrastructure.

- Exhaustive and randomized arithmetic verification.

- Investigation of pipeline synchronization failures.

- Experimental pipeline-placement study.

- Greedy Column Reduction multiplier architecture.

- Implementation of a broad range of carry-propagate adders.

- FPGA benchmarking under common conditions.

- Investigation of sparse prefix architectures.

- Investigation of higher-radix prefix structures.

- Analysis of FPGA-specific synthesis behavior.

- Identification of architecture-versus-technology interactions.

34. Final Conclusion

The internship began as an investigation into multiplier architectures and evolved into a broader study of arithmetic datapath architecture.

The first phase established multiple multiplier structures.

The second phase demonstrated that pipelining can dramatically change the achievable operating frequency, but also introduced new problems involving register overhead and temporal synchronization.

The third phase investigated the final carry-propagation bottleneck and demonstrated that the choice of final adder can substantially alter system-level behavior.

The most important conclusion is that high-speed digital arithmetic cannot be optimized by looking at a single metric such as theoretical gate count or logic depth.

Actual performance emerges from the interaction between:

- architecture,

- logic depth,

- fan-out,

- routing,

- registers,

- synthesis,

- technology primitives,

- and physical implementation.

The project therefore establishes a strong foundation for the next stage of development, extending the study from individual arithmetic architectures toward the integration of these building blocks into higher-level digital signal-processing systems. The results provide a practical basis for continued investigation into performance, efficiency, scalability, and implementation considerations for future DSP-oriented hardware.

35. Future Work

The completed work provides a foundation for several possible directions of further development. Future work may focus on:

- Integration of the studied arithmetic units into larger DSP-oriented datapaths

- Development of arithmetic-intensive processing blocks

- Evaluation of the architectures under additional workloads and operating conditions

- Investigation of numerical precision and arithmetic-range requirements

- Further optimization of area, timing, and power characteristics

- Exploration of additional arithmetic and reduction architectures

- Evaluation of the designs using ASIC-oriented implementation flows

- Characterization across different semiconductor technologies and implementation conditions

- Investigation of alternative approaches to improving hardware efficiency and scalability

- Development of a more complete DSP-oriented hardware prototype

The long-term objective is to build upon the present arithmetic architecture and implementation study toward a scalable, configurable DSP hardware platform, while continuing to evaluate the trade-offs between performance, area, power, and implementation complexity.