INTERNSHIP RESEARCH JOURNEY AND UNEXPECTED RESULTS

A Record of What the Experiments Actually Taught

1. Introduction

This document records not merely the final results of the internship, but the reasoning process that produced them.

The formal report describes what was implemented and measured.

This document records something different:

What was expected, what actually happened, why the result was surprising, and what was learned from it.

This distinction is important because much of the value of the project came from investigating results that initially appeared to be wrong, strange, or contradictory.

2. The First Important Lesson — The Fastest Architecture Is Not Necessarily the Most Elegant One

A recurring assumption during the project was that a theoretically efficient architecture should automatically produce the best FPGA result.

The experiments repeatedly disproved this.

An architecture can have:

- lower theoretical complexity,

- fewer logic levels,

- better asymptotic behavior,

yet still perform worse after FPGA synthesis.

Why?

Because the FPGA is not an abstract Boolean machine.

It contains specific physical resources.

A Cyclone II FPGA has dedicated structures that influence how arithmetic is mapped.

Therefore:

Theoretical Architecture

↓

RTL

↓

Quartus

↓

FPGA-specific Mapping

↓

Actual Fmax

The last step can be dramatically different from what a purely theoretical analysis predicts.

3. The Greedy Column Reduction Surprise

The Greedy multiplier produced a surprisingly strong frequency.

The initial reaction was essentially:

"This looks too fast."

The result was independently verified through functional testing.

The important architectural difference was that the Custom Greedy structure did not require the same conventional final RCA used by architectures that first reduce the partial-product matrix to two rows.

Instead, it directly resolved product columns.

This changes the critical-path structure.

A conventional architecture behaves approximately as:

PPG

↓

Reduction Tree

↓

Two Rows

↓

Final RCA

↓

Product

The Custom Greedy architecture instead approaches:

PPG

↓

Column Reduction

↓

Resolved Product Bits

Therefore the final carry-propagation bottleneck can be avoided.

The result demonstrated that an architecture with more local hardware can outperform an architecture with fewer apparent operations if it eliminates a dominant critical path.

This established a recurring theme of the project:

Removing a bottleneck can be more valuable than minimizing total hardware.

4. The Pipeline Testbench Was Not the Only Problem

The first pipelined multiplier failures were initially suspected to be caused by the testbench.

That suspicion was reasonable.

A non-pipelined testbench expects:

Input

↓

Computation

↓

Output

whereas a pipelined multiplier behaves like:

Input(t)

↓

Pipeline

↓

Output(t+n)

Therefore the golden result must be delayed.

After the expected-value synchronization was corrected, however, incorrect results still appeared.

This was the turning point.

The arithmetic itself was not necessarily wrong.

The pipeline structure was wrong.

5. The Real Pipeline Bug

Consider:

PPG

│

├────────────── path A

│

↓

Adder

│

↓

REG

│

↓

path B

Suppose path A is not consumed until a much later stage.

If path A is allowed to remain unregistered while path B passes through registers, then eventually:

A = transaction 0

B = transaction 1

may arrive at the same adder.

The adder performs exactly the operation it was designed to perform.

But it is operating on the wrong generation of data.

Therefore:

The hardware can be logically correct and temporally incorrect.

This was one of the most important lessons of the internship.

6. Why the Fix Increased Register Count

The obvious fix was to register the otherwise-unused signals as well.

For example:

PPG

↓

REG

↓

REG

↓

REG

↓

Future Consumer

Even if a particular bit did not participate in the first two stages, it still needed to remain synchronized with the rest of the computation.

Thus the project encountered a second-order optimization problem:

The arithmetic architecture may be efficient, but temporal alignment can require additional storage.

This produced substantial register overhead, particularly in the Custom Greedy architecture because its computation is naturally column-oriented rather than a conventional horizontal reduction tree.

7. Every-Stage Pipelining Did Not Continue Improving Performance Indefinitely

Another surprising result came from comparing every-stage and every-two-stage pipelining.

The intuitive expectation was:

More registers

=

Shorter paths

=

Higher Fmax

This is only partly true.

Once the major combinational path has been divided sufficiently, other delays begin to matter:

- register clock-to-Q,

- setup time,

- routing,

- interconnect,

- clock distribution,

- FPGA placement,

- remaining combinational logic.

Therefore:

Fmax

↑

│           ________

│         /

│       /

│     /

│   /

│ /

└──────────────────→ Pipeline depth

The curve eventually begins to flatten.

This was interpreted as pipeline saturation.

8. The Final RCA Became the Enemy

Wallace and Dadda reduction trees solve one problem extremely well:

Reduce many partial products to a small number of rows.

But they do not eliminate the need to add those final rows.

Therefore:

Wallace/Dadda

↓

Two Rows

↓

Final CPA

If that CPA is an RCA, the final carry chain can become the dominant critical path.

This explains why additional pipelining inside the reduction tree eventually stopped producing proportional improvements.

The tree had already been shortened.

The remaining bottleneck was somewhere else.

This led naturally to Phase 3.

9. The Kogge-Stone Result

Kogge-Stone was attractive because its theoretical carry depth is logarithmic.

The architecture therefore provides a strong candidate for replacing the RCA.

However, Kogge-Stone introduces many interconnections.

This means that the theoretical advantage:

O(log N)

must be considered alongside:

- routing

- fan-out

- wire congestion

- FPGA mapping

The project therefore treated Kogge-Stone not simply as "the fastest adder," but as a point in a larger architectural design space.

10. The Sklansky/Ladner-Fischer Surprise

Theoretical analysis predicts differences between Sklansky and Ladner-Fischer.

Yet Quartus produced effectively identical results for the tested implementation.

This was initially surprising.

The explanation is synthesis.

Quartus is not required to preserve the exact structural organization described by the RTL.

It can transform the logic into another equivalent structure that maps efficiently to the FPGA.

Therefore two apparently different architectures can converge to essentially the same implementation.

This became a major lesson:

If two RTL descriptions synthesize to the same FPGA structure, their theoretical differences may disappear in the measured FPGA result.

11. Carry Select Looked Strange Too

Carry Select theoretically duplicates hardware.

One therefore expects a significant area cost.

Yet the FPGA resource result did not necessarily look like a simple "two times the hardware" relationship.

The explanation is that FPGA LUTs are not equivalent to individual ASIC gates.

Quartus can pack and optimize the logic.

Therefore the measured LE count cannot be interpreted directly as transistor count.

This reinforced the distinction:

FPGA LE

≠

ASIC gate count

≠

transistor count

12. Sparse Kogge-Stone — The Difference Between Bad RTL and Optimized RTL

The initial Sparse Kogge-Stone implementation generated more of the tree than was actually necessary.

The expectation was that Quartus would remove the unused structure.

Quartus did.

But relying on synthesis to clean up the architecture is not the same as designing the architecture correctly.

The implementation was therefore rewritten so that the RTL itself represented the intended sparse structure.

This produced a more meaningful architecture.

The result was a large reduction in logic utilization, with some speed loss due to the local RCA sections.

This was a valuable design lesson:

A good RTL designing should generate the intended architecture directly rather than depending on downstream synthesis to repair inefficient structural descriptions.

13. Sparse Brent-Kung Was Even More Interesting

The first sparse Brent-Kung correction mechanism produced ripple behavior.

The problem arose because sparse prefix generation does not necessarily produce every intermediate carry.

Missing intermediate carry relationships created a dependency chain.

The correction stage effectively behaved like:

27

↓

23

↓

19

↓

15

which recreated a ripple-like path.

The architecture was then modified so that the Brent-Kung backward tree continued far enough to generate the required sparse terms.

The modified tree used fewer logic elements:

801 → 793 LE

but its measured Fmax decreased:

172.74 MHz → 159.87 MHz

This appears contradictory until FPGA implementation is considered.

The short ripple path may have been mapped into dedicated FPGA carry hardware.

The tree correction required more general routing.

Thus:

A theoretically more parallel path can be slower than a short ripple path on an FPGA if the FPGA has dedicated carry resources.

This is one of the most valuable experimental observations from the project.

14. Manchester Carry — A Technology Mismatch

Manchester Carry is an excellent example of an architecture whose usefulness depends heavily on implementation technology.

The tested FPGA implementation produced low resource usage but poor frequency.

The architecture is more naturally understood as transistor-level logic.

Potential implementations using:

- pass-transistor logic,

- transmission gates,

- dynamic logic,

could produce a completely different result.

Therefore the FPGA experiment should be interpreted as:

"Manchester Carry is poorly matched to this FPGA implementation."

It should NOT be interpreted as:

"Manchester Carry is intrinsically a bad adder."

15. Radix-4 Kogge-Stone

The Radix-4 implementation demonstrated another interesting direction.

Higher-radix prefix structures can reduce the number of logical stages.

For 16 bits:

Radix-2:

log₂(16) = 4 stages

Radix-4:

log₄(16) = 2 stages

The measured result was strong in the tested FPGA environment.

More importantly, the experiment opened a future research direction:

Radix-2

↓

Radix-4

↓

Radix-8

↓

Radix-16

Higher-radix structures may be especially interesting when custom ASIC cells are available.

16. The Fundamental Lesson

The internship started with a relatively simple question:

"Which multiplier is fastest?"

The experiments gradually transformed that question into something much deeper:

"Which architecture is fastest under a particular implementation technology, physical constraint, pipeline structure, synthesis system, and performance objective?"

That is a much more meaningful engineering question.

17. The Architecture × Technology Principle

The project repeatedly demonstrated:

ARCHITECTURE

×

IMPLEMENTATION TECHNOLOGY

×

SYNTHESIS

×

PHYSICAL RESOURCES

=

ACTUAL PERFORMANCE

This explains many of the apparently contradictory observations.

An architecture cannot be judged solely from a textbook diagram.

It must eventually be implemented.

18. What the Internship Actually Achieved

The project did not simply produce several multipliers.

It produced a methodology for investigating arithmetic architectures.

The completed framework allows architectures to be:

Designed

↓

Verified

↓

Pipelined

↓

Synthesized

↓

Measured

↓

Compared

↓

Explained

That methodology is arguably more important than any single Fmax number.

19. Transition to the Next Stage

At the conclusion of the internship, the research prototype contains the fundamental arithmetic building blocks required for a future DSP accelerator:

Multipliers

+

Adders

+

Pipelining

+

Verification

↓

Arithmetic Datapath

The next stage is therefore no longer primarily about asking:

"Which multiplier should I use?"

Instead, the question becomes:

"How do I combine these arithmetic primitives into a useful DSP compute engine?"

That marks the transition from the internship research prototype toward the long-term DSP architecture.

20. Final Reflection

The most valuable discoveries were not necessarily the results that matched expectations.

The most valuable results were the ones that initially looked wrong.

The unexpectedly fast Custom Greedy multiplier led to investigation of final-adder elimination.

The incorrect pipelined multiplier led to understanding temporal synchronization.

The limited improvement from additional registers led to the concept of pipeline saturation.

The strange Sparse Brent-Kung result demonstrated the importance of FPGA dedicated carry resources.

The identical Sklansky and Ladner-Fischer results demonstrated the power of synthesis optimization.

The Manchester Carry result demonstrated the difference between FPGA and transistor-level architectures.

The Sparse Kogge-Stone correction demonstrated the importance of designing the intended architecture directly.

Together, these observations transformed the project from an implementation exercise into an experimental study of digital architecture.

That is the real outcome of the internship.