FINAL INTERNSHIP CONTRIBUTIONS

Summary of Technical Work

1. Multiplier Architecture Investigation

Implemented and investigated multiple multiplier architectures, including:

- Braun

- Row Adder

- Carry Save

- Wallace

- Dadda

- Booth

- Baugh-Wooley

- Vedic

- Greedy Column Reduction

The architectures were evaluated using a common structural RTL and FPGA benchmarking methodology.

2. Pipelined Multiplier Framework

Implemented configurable pipeline strategies including:

- input pipelining,

- PPG pipelining,

- every-stage pipelining,

- every-two-stage pipelining,

- custom pipeline placement,

- output pipelining.

The work investigated the relationship between:

- pipeline depth,

- Fmax,

- delay,

- register utilization,

- combinational logic,

- and synchronization.

3. Pipeline Verification

Developed verification methodology for pipelined arithmetic circuits.

The verification system incorporated delayed golden-model results to account for pipeline latency.

A significant synchronization bug was identified in which signals entering the same computational stage had crossed different numbers of pipeline boundaries.

The issue was traced to unequal signal latency rather than incorrect arithmetic logic.

The datapath was corrected so that all signals required at a stage were synchronized.

4. Custom Greedy Multiplier

Developed and experimentally verified a Custom Greedy Column Reduction multiplier.

The architecture differs from conventional Wallace/Dadda reduction because it progressively resolves product columns rather than reducing the entire matrix to two rows followed by a conventional final RCA.

This creates a potentially important architectural trade-off:

More local reduction hardware

↓

Avoid final RCA

↓

Potentially higher speed

The architecture demonstrated strong FPGA performance and became one of the most significant architectural findings of the project.

5. Final Adder Investigation

Implemented and tested a broad family of carry-propagation architectures.

These included:

- RCA

- CLA

- Carry Skip

- Carry Select

- Kogge-Stone

- Brent-Kung

- Han-Carlson

- Sklansky

- Ladner-Fischer

- Knowles

- Sparse Kogge-Stone

- Sparse Brent-Kung

- Ling

- Manchester Carry

- Radix-4 Kogge-Stone

6. FPGA Benchmarking

The primary benchmarking platform was:

Quartus II 13.0sp1

Target FPGA:

Cyclone II DE2 — EP2C35F672C7

The adder comparison was performed within a 16×16 pipelined Wallace multiplier using:

- input register,

- every-two-stage pipelining,

- output register.

This common environment allowed the adders to be compared at system level rather than only as isolated blocks.

7. Experimental Discoveries

Several important observations were made:

Pipeline saturation

Additional pipeline registers did not always produce proportional Fmax improvements.

Final adder bottleneck

Wallace/Dadda reduction can leave the final carry-propagate adder as a critical path.

FPGA-specific carry optimization

Short ripple paths can sometimes outperform theoretically more sophisticated tree structures because of dedicated FPGA carry resources.

Synthesis convergence

Structurally different RTL architectures can synthesize into effectively identical FPGA implementations.

ASIC/FPGA distinction

Architectures such as Manchester Carry and higher-radix prefix structures may require transistor-level ASIC implementation for a fair evaluation.

Register overhead

Pipeline synchronization can require registers for signals that are not immediately consumed, producing significant storage overhead.

8. Research Methodology

The project established the following methodology:

Architecture

↓

Structural RTL

↓

Functional Verification

↓

Pipeline Integration

↓

Synthesis

↓

FPGA Implementation

↓

Measurement

↓

Unexpected Result

↓

Investigation

↓

Architectural Explanation

This methodology is reusable for future arithmetic and DSP research.

9. Internship Outcome

The internship successfully established a research prototype for high-speed arithmetic datapaths.

The completed work provides:

- multiplier architectures,

- pipelining techniques,

- verification infrastructure,

- adder architectures,

- FPGA benchmarking,

- architectural analysis,

- and a foundation for future DSP accelerator development.

The work therefore concludes the initial research-prototype phase and provides the technical foundation for the next stage: integration of these arithmetic components into a dedicated DSP compute architecture.